## SIMATSENGINEERING

### ASSESSMENTTOOL2

COURSENAME:COMPUTERNETWORKS COURSE CODE: CSA07

COURSEOUTCOMECOVERED

CO2: Configure IP addressing schemes,subnetting,and basic routing techniques using simulation tool, for efficient network design and topology.(BL3)

AssessmentToolWeightage: 30%

QUESTIONS: 5	Total Marks:50

AnnexureA

# Scenario-Based

Codeof Conduct:

I   RITESH.Y(192512100)  certifythatthissubmissionismyoriginalworkand that I have adhered to the guidelines specified for this assessment.

Iunderstandthatanyviolationofacademicintegrityruleswillresultindisciplinary action.

Signatureofthe student:

AnnexureA

# Scenario-Based

- A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administratorhasbeenallocatedtheIPaddressblock192.168.100.0/24.Basedon the expected number of devices, the following subnet masks have already been assigned:

SchoolofComputing→/25 SchoolofElectronics→ /26

SchoolofMechanicalEngineering→/27

Asthenetworkengineer,designthesubnetallocationbyperformingthefollowing tasks:

  - Determinethenetworkaddressandbroadcastaddressforeachdepartment.

  - IdentifythevalidhostIPaddressrangeforeachsubnet.

  - Calculatethenumberofusablehostaddressesavailableineachsubnet.

  - DeterminetheremainingunusedIPaddressrangewithinthegivennetwork block.

- A multinational software companyis expandingits headquarters andhas been assignedtheprivateIPblock10.10.0.0/16.Differentdepartmentsrequiredifferent subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24 Quality Assurance → /25 Human Resources → /26 ExecutiveManagement→/27

Asthenetworkadministrator,performthefollowingtasks:

  - Determinethenetworkaddressandbroadcastaddressforeach department.

  - IdentifythevalidrangeofhostIPaddresses.

  - Calculatethenumberofusablehostsineachsubnet.

  - Identify the unused IP address range remaining within the allocated IP block.

- A smart hospital is migrating its network infrastructure from IPv4 to IPv6 tosupportthousandsofconnectedmedicaldevicesandIoTsensors.Thehospitalhas been assigned the IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

Asthenetworkengineerresponsibleforthemigration,performthefollowing:

  - ConvertthegivenIPv6addressintoitscompressednotation.

  - Expand	the	compressed	address	back	into	its	full	hexadecimal representation.

  - Identifythenetworkprefixusingthe/64prefixlength.

  - Calculatethetotalnumberofhostaddressessupportedwithinthesubnet.

  - thetotalnumberofpossiblehostaddressesavailablewithinthisnetwork.

- AnInternetServiceProvider(ISP)connectsmultiplecustomernetworks through a core router. The router maintains the following routing table entries: 192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

ApacketarrivesattherouterwiththedestinationIPaddress:

192.168.2.45

Astheroutingengineer,determine:

  - Thedestinationsubnettowhichthepacketbelongs.

### Thematchingroutingtableentry.

  - Thenext-hopnetwork/interfaceusedtoforwardthepacket.

  - Thedefaultroutethatshouldbeusedifthedestinationdoesnotmatchany routing table entry.

  - A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

AdministrationLAN→192.168.10.0/26

FinanceLAN→192.168.10.64/26

Therouterinterfacesareconfiguredwith:

AdministrationGateway→192.168.10.1

FinanceGateway→192.168.10.65

Asthenetworkadministrator,performthefollowingtasks:

    - Determinethenetworkaddressandbroadcastaddressforeach LAN.

    - IdentifythevalidhostIPaddressrange.

    - SuggestsuitableIPaddressesfor threecomputersineachLAN.

    - IdentifythedefaultgatewaythathostsineachLANshoulduseforcommunication.

# Question 1

Scenario Understanding

The Smart Campus has been assigned the IPv4 network 192.168.100.0/24. Different departments require different subnet sizes based on the expected number of devices. Variable Length Subnet Masking (VLSM) is used to efficiently divide the address space while minimizing wastage.

### Step 1: Subnet Allocation

| Department | Subnet Mask | Network Address | First Host | Last Host | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- | --- | --- |
| School of Computing | /25 | 192.168.100.0 | 192.168.100.1 | 192.168.100.126 | 192.168.100.127 | 126 |
| School of Electronics | /26 | 192.168.100.128 | 192.168.100.129 | 192.168.100.190 | 192.168.100.191 | 62 |
| School of Mechanical Engineering | /27 | 192.168.100.192 | 192.168.100.193 | 192.168.100.222 | 192.168.100.223 | 30 |

### Step 2: Remaining Address Space

Unused subnet:

- Network Address: 192.168.100.224/27

- Host Range: 192.168.100.225 – 192.168.100.254

- Broadcast Address: 192.168.100.255

### Decision and Justification

The largest subnet (/25) is assigned first, followed by /26 and /27, ensuring efficient utilization of the available IP address space. The remaining /27 block can be reserved for future expansion, guest Wi-Fi, or network management devices. This design minimizes address wastage while allowing scalability.

# Question 2

Scenario Understanding

The multinational company has been allocated the private network 10.10.0.0/16. Each department has different host requirements, so VLSM is used to allocate subnet sizes according to departmental needs.

### Step 1: Subnet Details

| Department | Prefix | Network Address | First Host | Last Host | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- | --- | --- |
| Software Development | /24 | 10.10.0.0 | 10.10.0.1 | 10.10.0.254 | 10.10.0.255 | 254 |
| Quality Assurance | /25 | 10.10.1.0 | 10.10.1.1 | 10.10.1.126 | 10.10.1.127 | 126 |
| Human Resources | /26 | 10.10.1.128 | 10.10.1.129 | 10.10.1.190 | 10.10.1.191 | 62 |
| Executive Management | /27 | 10.10.1.192 | 10.10.1.193 | 10.10.1.222 | 10.10.1.223 | 30 |

### Remaining IP Address Space

Remaining range:

10.10.1.224 – 10.10.255.255

### Decision and Justification

The address allocation satisfies current departmental requirements and leaves a significant portion of the /16 network unused. This remaining address space can support future departments, branch offices, servers, or expansion without requiring a redesign of the network.

# Question 3

Scenario Understanding

The hospital is migrating from IPv4 to IPv6 to support thousands of medical devices and IoT sensors. IPv6 provides a much larger address space, simplifying future expansion.

### Solution

Given Address

2001:0db8:abcd:0000:0000:0000:0000:0000/64

### 1. Compressed Address

2001:db8:abcd::/64

### 2. Expanded Address

2001:0db8:abcd:0000:0000:0000:0000:0000/64

### 3. Network Prefix

2001:db8:abcd:0000::/64

### 4. Total Host Addresses

A /64 IPv6 subnet provides

2⁶⁴ = 18,446,744,073,709,551,616 addresses

### Decision and Justification

A /64 prefix is the standard IPv6 subnet size. It supports automatic address configuration (SLAAC), ensures compatibility with modern networking standards, and provides sufficient addresses for all present and future hospital devices.

# Question 4

Scenario Understanding

The ISP router forwards incoming packets based on the destination IP address using its routing table. Longest-prefix matching is used to identify the correct destination network.

### Routing Table

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

Destination IP:

192.168.2.45

### Solution

Destination Subnet

192.168.2.0/24

Matching Route

192.168.2.0/24

Next-Hop Interface

The interface connected to the 192.168.2.0/24 network.

Default Route

0.0.0.0/0

### Decision and Justification

Since the destination address belongs to the 192.168.2.0/24 subnet, the router forwards the packet through the interface connected to that subnet. If no matching route existed, the packet would be forwarded through the default route (0.0.0.0/0).

# Question 5

Scenario Understanding

A company has divided its network into two LANs connected through a Layer-3 router. Each LAN has its own subnet and gateway, allowing efficient communication while reducing broadcast traffic.

### Administration LAN

- Network Address: 192.168.10.0/26

- Broadcast Address: 192.168.10.63

- Host Range: 192.168.10.1 – 192.168.10.62

- Default Gateway: 192.168.10.1

Suggested IP Addresses

- PC1 → 192.168.10.2

- PC2 → 192.168.10.3

- PC3 → 192.168.10.4

### Finance LAN

- Network Address: 192.168.10.64/26

- Broadcast Address: 192.168.10.127

- Host Range: 192.168.10.65 – 192.168.10.126

- Default Gateway: 192.168.10.65

Suggested IP Addresses

- PC1 → 192.168.10.66

- PC2 → 192.168.10.67

- PC3 → 192.168.10.68

### Decision and Justification

The router gateways (192.168.10.1 and 192.168.10.65) enable communication within each subnet and between the two LANs. Assigning host IP addresses immediately after the gateway simplifies network management and follows standard IP addressing practices. The /26 subnet supports 62 usable hosts per LAN, providing room for future expansion.

RUBRICSFORCALCULATION

| ScenarioBased Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understandin g of the scenario and context | Showsgood understandi ng with minor gaps | Partial understandin g;misseskey aspects | Misinterpret s or fails to understand thescenario |
| Identification ofKeyIssues | 20% | Accurately identifiesall relevant issues and factors involved | Identifies most key issueswith minor omissions | Identifies limited issues;some irrelevant points | Unable to identifykey issues |
| Applicationof Concepts | 25% | Applies appropriate concepts effectivelyto addressthe scenario | Applies concepts withminor errors | Limited or partially correct application | Unableto apply relevant concepts |
| Decision Making | 20% | Makeswell-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makesweak decisions with limited justification | No clear decisions made or rationale provided |
| Clarityof Response | 15% | Response is clear, structured, andlogically presented | Generally clearwith minor issues | Somewhat unclear or poorly structured | Disorganize d and difficult to understand |