# SIMATSENGINEERING

## ASSESSMENTTOOL1

COURSENAME:	CSA07COURSECODE:Computer Networks

COURSEOUTCOMECOVERED

CO1:ApplythefunctionsofOSIlayerinenablingreliabledatatransmissionacross networks. (BL2, BL3)

AssessmentToolWeightage: 40%

AnnexureA

SHORTANSWERTEST

Codeof Conduct:

I  RITESH.Y/192512100	(Name/RegNo)certifythatthissubmissionismyoriginalworkand that I have adhered to the guidelines specified for this assessment.

Iunderstandthatanyviolationofacademicintegrityruleswillresultindisciplinary action.

Signatureofthe student:

AnnexureA

ShortAnswerTest

- Acompanyneedstotransfera150MBdesignfilefromaclientcomputertoa remoteserveroveranetworkwithabandwidthof50Mbps.TheTransportLayer dividesthefileinto1500-bytesegments,whiletheDataLinkLayerencapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

ANSWER:

Given:

- File size = 150 MB

- Segment size = 1500 bytes

- Data Link Layer overhead = 40 bytes/frame

- Bandwidth = 50 Mbps

Answer:

- Number of segments: 104,858

- Number of frames: 104,858 (one frame per segment)

- Total overhead: 4,194,320 bytes (≈ 4 MB)

- Transmission time: ≈ 25.84 seconds (ignoring propagation delay)

Reliability:

- The Transport Layer divides the file into segments, assigns sequence numbers, and ensures reliable delivery using acknowledgments and retransmissions.

- The Data Link Layer encapsulates each segment into a frame, adds headers and trailers for addressing and error detection (CRC), ensuring error-free communication over each network link.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each framecarries1024bytesofuserdata.Ifthesendertransmits48frames,calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

ANSWER:

Given:

- Sliding window size = 6 frames

- Total frames = 48

- Data per frame = 1024 bytes

Answer:

- Number of transmission windows: 48÷6=848 \div 6 = 848÷6=8 windows

- Total retransmissions: 8 frames (one lost frame in each window)

Flow Control:
The Transport Layer uses flow control to regulate the sender's transmission rate so that the receiver is not overloaded. This improves communication efficiency by preventing data loss, reducing retransmissions, and ensuring reliable data transfer.

- A 12,000-byte IP packet is transmitted through a router where the Maximum TransmissionUnit(MTU)is1500bytes.Assumingeachfragmentrequiresa20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

ANSWER:

Given:

- IP packet size = 12,000 bytes

- MTU = 1500 bytes

- IP header per fragment = 20 bytes

Answer:

- Number of fragments: 9

- Total header overhead: 9 × 20 = 180 bytes

Network Layer:
The Network Layer fragments large packets to match the MTU of the network. Each fragment contains its own IP header with identification, offset, and flag fields. At the destination, these fragments are reassembled in the correct order to recover the original packet, ensuring successful delivery.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each framecarries1024bytesofuserdata.Ifthesendertransmits48frames,calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

ANSWER:

Given:

- Sliding window size = 6 frames

- Total frames = 48

- Data per frame = 1024 bytes

Answer:

- Number of transmission windows: 8

- Total retransmissions: 8 frames (one lost frame in each window)

Flow Control:
Flow control regulates the rate of data transmission between the sender and receiver. It prevents the receiver from being overloaded, reduces data loss and retransmissions, and improves the overall efficiency and reliability of communication.

- A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amountofdatathatmustberetransmittedafterrecovery.Explaintheimportance of synchronization in the Session Layer.

ANSWER:

Given:

- Video file size = 600 MB

- Checkpoint after every 60 MB

- Failure after 390 MB

Answer:

- Checkpoints created before failure: 6 (at 60, 120, 180, 240, 300, and 360 MB)

- Data to be retransmitted: 390 − 360 = 30 MB

Session Layer:
The Session Layer provides synchronization by creating checkpoints during data transfer. If a failure occurs, communication resumes from the last checkpoint instead of restarting from the beginning, saving time and improving transmission efficiency.

- Beforetransmission,thePresentationLayercompressesa900MBfileby40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculatethecompressedfilesize,theencryptedfilesize,andthetotalpercentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

ANSWER:

Given:

- Original file size = 900 MB

- Compression = 40%

- Encryption overhead = 5%

Answer:

- Compressed file size: 540 MB

- Encrypted file size: 567 MB

- Total reduction compared to original: 37%

Presentation Layer:
The Presentation Layer compresses data to reduce transmission time and bandwidth usage. It also encrypts data to provide confidentiality and secure communication during transmission.

- An FTP application transfers 3 GB of data over a network with an effective throughputof120Mbps.TheApplicationLayerdividesthedataintorequestsof 3MBeach.Calculatethetotalnumberofrequestsgeneratedandestimatethetotal transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

ANSWER:

Given:

- Data size = 3 GB = 3000 MB

- Request size = 3 MB

- Throughput = 120 Mbps

Answer:

- Number of requests: 3000 ÷ 3 = 1000 requests

- Transmission time: ≈ 200 seconds (about 3 minutes 20 seconds)

Application Layer:
The Application Layer provides services such as file transfer (FTP), error handling, and user authentication. It ensures reliable communication between the user and the network by managing requests and responses efficiently.

- A packet experiences the following processing delays at different OSI layers: ApplicationLayer=4ms,TransportLayer=3ms,NetworkLayer=5ms,Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

ANSWER:

Given:

- Application Layer = 4 ms

- Transport Layer = 3 ms

- Network Layer = 5 ms

- Data Link Layer = 2 ms

- Physical Layer = 1 ms

- Propagation delay = 18 ms

- Transmission delay = 12 ms

Answer:

- Total processing delay: 4+3+5+2+1=154 + 3 + 5 + 2 + 1 = 154+3+5+2+1=15 ms

- Total end-to-end delay: 15+18+12=4515 + 18 + 12 = 4515+18+12=45 ms

OSI Layer Contribution:
Each OSI layer performs a specific function: the Application Layer provides user services, the Transport Layer ensures reliable data delivery, the Network Layer handles routing, the Data Link Layer manages framing and error detection, and the Physical Layer transmits data as signals over the communication medium.

- A network transfers 80 MB of useful data using frames of 1600 bytes, where eachframecontains80bytesofprotocoloverheadaddedbydifferentOSIlayers.

Calculatethetotalnumberofframesrequired,thetotaloverheadtransmitted,and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

ANSWER

Given:

- Useful data = 80 MB

- Frame size = 1600 bytes

- Protocol overhead = 80 bytes/frame

Answer:

- Number of frames: 52,429

- Total overhead: 4,194,320 bytes (≈ 4 MB)

- Transmission efficiency: 95%

Protocol Overhead:
Protocol overhead consists of headers and trailers added by different OSI layers for addressing, error detection, and control. While it ensures reliable communication, excessive overhead reduces transmission efficiency and increases bandwidth usage.

- A hospital transfers a patient's medical record of 240 MB through an IoT-enabledhealthcarenetwork.ThePresentationLayercompressesthedataby30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed filesize,thetotal numberof segments transmitted,thetotalprotocol overheadintroduced,andthefinalamountofdatatransmittedacrossthePhysical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

ANSWER:

Given:

- Original file size = 240 MB

- Compression = 30%

- Segment size = 1400 bytes

- Data Link Layer overhead = 60 bytes/frame

Answer:

- Compressed file size: 168 MB

- Total number of segments: 125,829

- Total protocol overhead: 7,549,740 bytes (≈ 7.2 MB)

- Final data transmitted: ≈ 175.2 MB

OSI Layer Working:
The Presentation Layer compresses and encrypts data, the Transport Layer divides it into segments and ensures reliable delivery, the Data Link Layer adds frames with error detection, and the Physical Layer transmits the data over the network. Together, these layers provide efficient, secure, and reliable communication.

RUBRICSFORCALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level3– Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g ofConcepts | 25% | Demonstrates a thorough, accurate understandin g ofconcepts | Good understanding withminorgaps | Basic understanding with some misconception s | Poor understanding; major misconception s |
| Explanation | 25% | Detailed,well-explained answers with relevant examples | Clear explanation withadequate details | Limited explanation; lacks depth | Poororunclear explanation |
| Application | 20% | Effectively applies conceptswith strong analytical ability | Applies conceptswith minor errors | Limited applicationwith weak analysis | Noapplication or incorrect analysis |
| Organization | 15% | Well-structured answerswith logical flow | Organizedwith minorissuesin flow | Basicstructure,lacks clarity | Poor organization, incoherent answers |

|  |  | and coherence |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Accuracy | 10% | Completely accurateand covers all required points | Mostlyaccurate with minor omissions | Partially correct with missingpoints | Incorrect answerswith major gaps |
| Clarity | 5% | Clear, neat, professional presentation | Understandabl e with minor issues | Limitedclarity, readability issues | Poorlywritten, difficult to understand |